MORE TEA

Hardware Components Report

Linorobot2-Based Tour and Navigation Robot

Garage@EEE Enhancements

September 2026

# Contents

This report covers the More Tea physical platform used with Linorobot2. Navigation is the first acceptance target. Perception and manipulation hardware is a later extension and must not delay the mobile base.

| Section | Page |
| --- | --- |
| 1. System architecture and compute | 3 |
| 2. Mobile base and navigation hardware | 4 |
| 3. Power, chassis and user interface | 5 |
| 4. Later perception and manipulation | 6 |
| 5. Interfaces, safety and acceptance | 7 |

## How to read the status labels

Confirmed: A requirement already stated for the project.

Project baseline: The current planned platform or recommended starting point.

Design-dependent: Quantity or exact model must be selected after measurement and review.

Later extension: Not required for the minimum working system; add only after the core robot passes acceptance.

# 1. System architecture and compute

More Tea separates high-level ROS 2 navigation from low-level motion control. The Jetson runs Linorobot2, Nav2, mapping, localization and the touchscreen interface. The custom ESP32 PCB runs micro-ROS, controls the motor drivers, reads wheel encoders and returns feedback to ROS 2.

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | Jetson Orin Nano, 8 GB | Runs ROS 2, Linorobot2, ros2_control integration, SLAM Toolbox, AMCL, Nav2, RViz 2 and later perception. | Project baseline |
| 1 | NVMe storage | Holds the operating system, containers, ROS workspaces, maps, logs and datasets. | Project baseline |
| 1 | Active cooling | Maintains stable Jetson performance during navigation and later perception workloads. | Project baseline |
| 1 | Custom ESP32 PCB | Provides the low-level motor, encoder, watchdog and emergency-stop interface. | Confirmed |
| 1 | micro-ROS transport connection | Links ESP32 topics/services to the Jetson by the selected serial or network transport. | Confirmed |
| As required | USB hub or protected serial interfaces | Provides reliable connections for LiDAR, IMU, touchscreen and controller devices. | Design-dependent |
| As required | Debug/programming interface | Allows ESP32 firmware flashing, serial logs and recovery during bench testing. | Project baseline |

Required platform relationship More Tea is based on Linorobot2 and uses ESP32 plus micro-ROS. The Jetson is the ROS 2 host; the ESP32 is the real-time controller. Neither part should be omitted from this hardware plan.

# 2. Mobile base and navigation hardware

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 4 | 12 V geared DC motors with encoders | Provides four-wheel skid-steer motion and wheel feedback for odometry. | Confirmed |
| 4 | Wheels | Supports the four-wheel mobile base; diameter must be measured for Linorobot2 parameters. | Confirmed |
| 1 set | Motor-driver system | Drives all four motors from the ESP32. Channel count and current rating must match measured stall current. | Design-dependent |
| 4 | Encoder input channels | Returns wheel rotation for velocity control and odometry. | Confirmed |
| 1 | RPLIDAR S2 | Provides the 360-degree LaserScan used for mapping, localization and obstacle detection. | Project baseline |
| 1 | IMU | Provides angular velocity and acceleration for odometry fusion and motion diagnostics. | Project baseline |
| 1 | Emergency-stop switch | Removes or disables motor drive independently of ROS 2 and the Jetson. | Confirmed |
| 1 | Mobile chassis | Carries the compute, sensors, power system, touchscreen and later arm while maintaining safe clearance. | Confirmed |
| As required | LiDAR, IMU and encoder cabling | Provides restrained, labelled sensor and feedback connections with stable device identification. | Design-dependent |

## Geometry that must be measured

Wheel diameter or radius, effective track width and wheelbase.

Encoder counts per wheel revolution after gearbox effects.

LiDAR and IMU positions and orientations relative to the base frame.

Ground clearance, sensor blind zones and the final centre of gravity.

# 3. Power, chassis and user interface

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | 12 V battery system | Supplies the drive motors and feeds suitable regulated branches for the remaining hardware. | Project baseline |
| As required | DC-DC regulators | Provides stable voltage rails for the Jetson, ESP32, sensors, touchscreen and later accessories. | Design-dependent |
| 1 | 10-inch touchscreen | Provides destination selection, robot status and operator controls. | Confirmed |
| 1 | Main power switch | Provides complete system isolation. | Project baseline |
| As required | Fuses or circuit protection | Protects the battery, motor, compute and accessory branches. | Project baseline |
| 1 | Power-distribution board or harness | Distributes labelled power branches and supports safe maintenance. | Design-dependent |
| As required | Connectors, terminal blocks and cable restraint | Prevents loose or reversed connections and protects cables during robot motion. | Design-dependent |
| As required | Sensor and compute mounts | Holds the LiDAR level, secures the IMU, protects the Jetson and provides cooling airflow. | Confirmed |
| As required | Bumpers or protective body panels | Protects equipment and reduces contact risk in the tour environment. | Design-dependent |

Power rule Do not run the motors through the Jetson or ESP32 supply rail. Use separately protected motor and compute branches, confirm grounding and interface requirements, and verify the emergency stop under load.

# 4. Later perception and manipulation

These components are added only after More Tea can repeatedly map, localize, accept a touchscreen goal, navigate safely and stop correctly. The first manipulation feature must remain bounded and must not weaken navigation safety.

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | Camera or depth camera | Supports later target detection, recognition and target-frame generation. | Later extension |
| 1 | Robot arm | Carries out one approved, bounded manipulation task after navigation acceptance. | Later extension |
| 1 | Gripper or end effector | Interacts with the approved object for the selected manipulation task. | Later extension |
| As required | Arm actuators | Provides the required joints. Exact actuator type, torque and count follow the final arm design. | Design-dependent |
| If selected | STS3215 serial bus servos | May be used for the arm or gripper only if torque, speed, travel and load testing show they are suitable. | Design-dependent |
| If selected | Serial servo controller board | Interfaces a selected STS3215 chain to the Jetson or ESP32. | Design-dependent |
| 1 | Dedicated arm/servo power branch | Supplies manipulation hardware without disturbing navigation compute or motor-control power. | Design-dependent |
| As required | Arm mounting and stow hardware | Secures the arm, protects the centre of gravity and keeps it within a safe travel envelope. | Later extension |

Navigation-first gate MoveIt 2 and manipulation hardware begin only after the navigation acceptance gate passes. Until then, camera and arm work remains bench-only and cannot delay Linorobot2 integration.

# 5. Interfaces, safety and acceptance

## Primary hardware interfaces

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | Jetson to ESP32 micro-ROS link | Carries movement commands, encoder feedback, status and diagnostics between Linorobot2 and the controller. | Confirmed |
| 1 | ESP32 to motor drivers | Carries direction, PWM or speed commands and hardware enable/stop signals. | Confirmed |
| 4 | Encoders to ESP32 | Carries wheel feedback for control and odometry. | Confirmed |
| 1 each | LiDAR and IMU interfaces | Provides stable sensor data with known device names, frame names and timestamps. | Project baseline |
| 1 | Touchscreen to Jetson | Provides display, touch input and operator feedback. | Confirmed |
| 1 | Physical emergency-stop circuit | Stops motor drive without depending on software, network state or AI. | Confirmed |

## Hardware acceptance evidence

Complete bill of materials with exact models, quantities, ratings and datasheets.

Measured motor stall current and approved motor-driver, wire, connector and fuse ratings.

Verified ESP32 pin map, micro-ROS transport, encoder direction and command-timeout behaviour.

Stable LiDAR and IMU connections with documented mounting positions and orientations.

Successful emergency-stop test while the physical base is moving under controlled conditions.

Measured wheel geometry and encoder scale entered into the Linorobot2 configuration.

Power-up, shutdown, brownout, cable-disconnection and reconnection test records.

Navigation acceptance completed before any arm-and-camera feature is installed on the moving robot.

Open purchasing decisions The final motor-driver model, IMU model, battery capacity, regulator ratings, connector set, camera, arm, gripper and manipulation actuators remain subject to electrical and mechanical review.