TALKING ROBOT

Hardware Components Report

Moving Pet Bot and Stationary Desk Buddy

Garage@EEE Enhancements

September 2026

# Contents

This report covers the hardware needed for both talking-robot variants. Shared parts are listed once; mobile-base parts apply only to the Pet Bot, while stationary interaction hardware applies to the Desk Buddy.

| Section | Page |
| --- | --- |
| 1. Shared talking-robot platform | 3 |
| 2. Moving Pet Bot hardware | 4 |
| 3. Stationary Desk Buddy hardware | 5 |
| 4. Power, interfaces and safety | 6 |
| 5. Selection and acceptance checklist | 7 |

## How to read the status labels

Confirmed: A requirement already stated for the project.

Project baseline: The current planned platform or recommended starting point.

Design-dependent: Quantity or exact model must be selected after measurement and review.

Later extension: Not required for the minimum working system; add only after the core robot passes acceptance.

# 1. Shared talking-robot platform

Both robots use the same spoken-assistant concept: microphone input, speech recognition, the Garage@EEE knowledge system, a local LLM, speech synthesis and speaker output. Each physical robot needs its own installed compute and interaction hardware.

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 per robot | Raspberry Pi 5, 8 GB | Runs the shared local LLM, speech pipeline, retrieval system, display application and high-level integration. | Project baseline |
| 1 per robot | Storage | microSD card or suitable SSD for the operating system, models, knowledge files and logs. | Design-dependent |
| 1 per robot | Active cooling | Keeps the Raspberry Pi within a stable temperature range during sustained inference and audio processing. | Project baseline |
| 1 per robot | USB microphone or microphone array | Captures spoken questions and wake/input audio. | Confirmed |
| 1 set per robot | Amplified speaker system | Plays speech-synthesis output and status sounds at a usable volume. | Confirmed |
| 1 per robot | Display or touchscreen | Shows expressions, status, retrieved sources and basic controls. | Confirmed |
| As required | STS3215 serial bus servos | Provides expressive movement such as head, ear or arm motion. Final count follows the approved mechanism. | Project baseline |
| 1 per robot | Serial servo controller board | Connects the Raspberry Pi or ESP32 to the STS3215 serial bus and manages servo commands. | Project baseline |
| As required | Buttons and indicator LEDs | Provides power, mute, reset and simple status feedback without relying only on software. | Design-dependent |
| 1 per robot | Custom enclosure | Holds the compute, display, audio devices, servo hardware and wiring with service access. | Confirmed |

Shared software does not mean shared hardware. The Pet Bot and Desk Buddy each require their own microphone, speaker, compute, power and installed interface devices.

# 2. Moving Pet Bot hardware

The Pet Bot adds a four-wheel mobile base, obstacle sensing and table-edge sensing. The Raspberry Pi performs high-level processing; the ESP32 owns time-critical movement and safety behaviour.

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | ESP32 development board or custom PCB | Controls motor outputs, reads sensors, applies command timeout and carries out immediate stop behaviour. | Confirmed |
| 4 | Geared DC motors, Motor encoders ESP32 with DC Motor - Control Speed and Direction ¦ Random Nerd Tutorials | Drives the four-wheel skid-steer mobile base. | Confirmed |
| 4 | Wheels | One wheel per DC motor. | Confirmed |
| 1 or 2 | H-bridge motor-driver boards TB6612FNG H-Bridge with Arduino - Better Than L298N? Reading the encoder value of N20 motor using ESP32 - Hackster.io | Drives all four motors. Board count depends on channel count and measured combined stall current. | Design-dependent |
| 2 | Ultrasonic sensors | Detects obstacles in the front-left and front-right directions. | Confirmed |
| 2 | Downward-facing IR sensors | Detects the table edge at the front-left and front-right corners. | Confirmed |
| 1 | Single-lens camera | Supports person detection, recognition and visual tracking on the Raspberry Pi. | Project baseline |
| 1 | Physical emergency-stop control | Stops the drive system independently of the LLM and high-level software. | Project baseline |
| 1 | Motor battery – 12v | Supplies the four DC motors through the selected motor-driver arrangement. | Design-dependent |
| 1 | Regulated compute power source | Provides stable power to the Raspberry Pi and logic electronics. | Design-dependent |
| 1 | Mobile chassis and wheel mounts | Supports the motors, sensors, power system, Raspberry Pi, ESP32 and talking-robot enclosure. | Confirmed |
| Pet Bot sizing gate Do not choose the motor-driver model or battery solely from the nominal 12 V label. Measure or obtain the stall current of one motor, then size each driver channel, fuse, connector and wire for the real worst-case load. |  |  |  |

# 3. Stationary Desk Buddy hardware

The Desk Buddy does not need drive motors, motor drivers, ultrasonic sensors or table-edge sensors. Its hardware is centred on voice, display, expression and a stable desktop enclosure.

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | Raspberry Pi 5, 8 GB | Runs the shared LLM, voice, retrieval and user-interface software. | Project baseline |
| 1 | USB microphone or microphone array | Captures the user's spoken questions at the desk. | Confirmed |
| 1 set | Amplified speaker system | Plays the spoken response and feedback sounds. | Confirmed |
| 1 | Display or touchscreen | Shows the face, system status, sources and interaction controls. | Confirmed |
| As required | STS3215 serial bus servos Link: What STS3215 looks like Github library | Moves the approved expressive mechanism; the quantity follows the final head, ear or arm design. | Project baseline |
| 1 | Serial servo controller board | Sends position, speed and limit commands to the STS3215 bus. | Project baseline |
| 1 | Dedicated servo power supply | Supplies the servo chain without drawing servo current from the Raspberry Pi power rail. | Design-dependent |
| 1 | Mute or stop control | Lets a user silence audio and stop expressive motion immediately. | Project baseline |
| 1 | Stationary enclosure | Positions the screen, microphone and speakers while providing ventilation and maintenance access. | Confirmed |
| As required | Connectors, cable restraint and internal mounts | Keeps the desktop unit serviceable and prevents loose cables from loading the boards. | Design-dependent |
| Desk Buddy boundary STS3215 movement is expressive only. The Desk Buddy remains stationary and must not include the Pet Bot's four-wheel drive hardware. |  |  |  |

# 4. Power, interfaces and safety

## Required interfaces

| Qty. | Component | Purpose | Status |
| --- | --- | --- | --- |
| 1 | Raspberry Pi to ESP32 link | USB serial or UART connection for bounded Pet Bot commands, status and heartbeat messages. | Project baseline |
| 1 | Raspberry Pi to serial servo board link | USB or UART connection for STS3215 expression commands and feedback. | Project baseline |
| As required | Audio and display connections | USB/I2S/audio-amplifier and HDMI/DSI/USB links selected for the final devices. | Design-dependent |
| As required | Power-distribution board or wiring harness | Separates compute, motor and servo loads and provides labelled, serviceable connections. | Design-dependent |
| As required | Fuses or resettable protection | Protects battery branches and wiring from faults. | Project baseline |
| 1 per robot | Main power switch | Provides complete system isolation. | Project baseline |

## Non-negotiable integration rules

Do not power DC motors or STS3215 servos from the Raspberry Pi's 5 V rail.

Use separate, correctly regulated power branches for compute, drive motors and servos; share signal reference only where the chosen interface requires it.

Alternate the two ultrasonic triggers to reduce cross-interference.

Calibrate both IR sensors on the actual table surface and lighting. With no rear IR sensor, reverse motion must remain brief and controlled.

If a selected ultrasonic sensor returns a 5 V Echo signal, level-shift it before connecting to a 3.3 V ESP32 input.

The LLM may request a movement or expression, but it must never bypass the ESP32 stop logic, command timeout, physical stop or servo limits.

# 5. Selection and acceptance checklist

Record the exact model, supplier, electrical rating and quantity for every purchased part. A component is not approved merely because it appears in this report.

## Items still requiring final selection

Exact motor voltage, gearbox ratio, wheel size and stall current.

Motor-driver model and whether one four-channel board or two dual-channel boards are used.

Pet Bot battery capacity, compute regulator and protection ratings.

Microphone, speaker amplifier, speaker count and display model for each robot.

Number and placement of STS3215 servos for each approved expressive mechanism.

Serial servo controller model and servo power-supply voltage/current rating.

Whether Pet Bot motor encoders are required for the first accepted prototype.

## Minimum hardware acceptance evidence

Photographed and labelled wiring or connection diagram.

Measured idle and worst-case current for each power branch.

Pet Bot: four-motor drive, two-ultrasonic, two-IR and independent stop test.

Desk Buddy: microphone, speaker, display, servo-limit and mute/stop test.

Both robots: repeatable cold start, safe shutdown and device-reconnection check.

Final bill of materials with exact quantities and links or datasheets.